COURSE CODE: CSA1260

COURSE NAME: Computer Architecture for Performance Optimization

NAME: GANESHWAR S

REGISTER NUMBER:192421342

8-BIT ADDITION

EXP NO: 1

AIM:

To write an assembly language program to implement 8-bit addition using 8085 processor.

ALGORITHM:

1) Start the program by loading the first data into the accumulator.

2) Move the data to a register.

3) Get the second data and load it into the accumulator.

4) Add the two register contents.

5) Check for carry.

6) Store the value of sum and carry in the memory location.

7) Halt.

PROGRAM:

LDA 8050

MOV B, A

LDA 8051

ADD B

STA 8052

HLT

INPUT:

| ADDRESS | DATA |
| --- | --- |
| 8050 | 1 |
| 8051 | 2 |

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 8052 | 3 |

RESULT: Thus the program was executed successfully using 8085 processor simulator.

8-BIT SUBTRACTION

EXP NO: 2

AIM: To write an assembly language program to implement 8-bit subtraction using 8085 processor.

ALGORITHM:

1) Start the program by loading the first data into the accumulator.

2) Move the data to a register.

3) Get the second data and load it into the accumulator.

4) Subtract the two register contents.

5) Check for borrow.

6) Store the difference and borrow in the memory location.

7) Halt.

PROGRAM:

LDA 8000

MOV B, A

LDA 8001

SUB B

STA 8002

HLT

INPUT:

| ADDRESS | DATA |
| --- | --- |
| 8000 | 4 |
| 8001 | 5 |

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 8002 | 1 |

RESULT: Thus the program was executed successfully using 8085 processor simulator

8-BIT MULTIPLICATION

EXP NO: 3

AIM: To write an assembly language program to implement 8-bit multiplication using 8085 processor.

ALGORITHM:

1) Start the program by loading a register pair with the address of memory location.

2) Move the data to a register.

3) Get the second data and load it into the accumulator.

4) Add the two register contents.

5) Increment the value of the carry.

6) Check whether the repeated addition is over.

7) Store the value of product and the carry in the memory location.

8) Halt.

PROGRAM:

LDA 2200

MOV E, A

MVI D, 00

LDA 2201

MOV C, A

LXI H, 0000

BACK: DAD D

DCR C

JNZ BACK

SHLD 2202

HLT

INPUT:

| ADDRESS | DATA |
| --- | --- |
| 2200 | 4 |
| 2201 | 2 |

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 2202 | 8 |

RESULT: Thus the program was executed successfully using 8085 processor simulator.

8-BIT DIVISION

EXP NO: 4

AIM: To write an assembly language program to implement 8-bit division using 8085 processor.

ALGORITHM:

1) Start the program by loading a register pair with the address of memory location.

2) Move the data to a register.

3) Get the second data and load it into the accumulator.

4) Subtract the two register contents.

5) Increment the value of the carry.

6) Check whether the repeated subtraction is over.

7) Store the value of quotient and the reminder in the memory location.

8) Halt.

PROGRAM:

NOP

LDA 8500

MOV B, A

LDA 8501

MVI C, 00H

LOOP: CMP B

JC LOOP1

SUB B

INR C

JMP LOOP

LOOP1: STA 8502

MOV A, C

STA 8503

RST 1

HLT

INPUT:

| ADDRESS | DATA |
| --- | --- |
| 8500 | 2 |
| 8501 | 6 |

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 8502 | 0 |
| 8503 | 3 |

RESULT: Thus the program was executed successfully using 8085 processor simulator.

16-BIT ADDITION

EXP NO: 5

AIM:-

To write an assembly language program to implement 16-bit addition using 8085 processor.

ALGORITHM:-

1) Start the program by loading a register pair with address of 1st number.

2) Copy the data to another register pair.

3) Load the second number to the first register pair.

4) Add the two register pair contents.

5) Check for carry.

6) Store the value of sum and carry in memory locations.

7) Terminate the program.

PROGRAM:-

LDA 3050

MOV B, A

LDA 3051

ADD B

STA 3052

LDA 3053

MOV B, A

LDA 3054

ADC B

STA 3055

HLT

INPUT:-

| Address | Data |
| --- | --- |
| 3050 | 2 |
| 3051 | 3 |
| 3053 | 5 |
| 3054 | 5 |

OUTPUT:-

| Address | Data |
| --- | --- |
| 3052 | 5 |
| 3055 | 10 |

RESULT:-

Thus the program was executed successfully using 8085 processor simulator.

16-BIT SUBTRACTION

EXP NO: 6

AIM:-

To write an assembly language program to implement 16-bit subtraction using 8085 processor.

ALGORITHM:-

1) Start the program by loading a register pair with address of 1st number.

2) Copy the data to another register pair.

3) Load the second number to the first register pair.

4) sub the two register pair contents.

5) Check for carry.

6) Store the value of sum and carry in memory locations.

7) End.

PROGRAM:-

LHLD 2050

XCHG

LHLD 2052

MOV A, L

SUB E

STA 2054

MOV A, H

SBB D

STA 2055

HLT

INPUT:-

| Address | Data |
| --- | --- |
| 2050 | 2 |
| 2052 | 3 |

OUTPUT:-

| Address | Data |
| --- | --- |
| 2054 | 1 |
| 2055 | 1 |

RESULT:-

Thus the program was executed successfully using 8085 processor simulator.

16-BIT MULTIPLICATION

EXP NO: 7

AIM:-

To write an assembly language program to implement 16-bit multiplication using 8085 processor.

ALGORITHM:-

1) Load the first data in HL pair.

2) Move content of HL pair to stack pointer.

3) Load the second data in HL pair and move it to DE.

4) Make H register as 00H and L register as 00H.

5) ADD HL pair and stack pointer.

6) Check for carry if carry increment it by 1 else move to next step.

7) Then move E to A and perform OR operation with accumulator and register D.

8) The value of operation is zero, then store the value else go to step

PROGRAM:-

LHLD 2050

SPHL

LHLD 2052

XCHG

LXI H,0000H

LXI B,0000H

AGAIN: DAD SP

JNC START

INX B

START: DCX D

MOV A, E

ORA D

JNZ AGAIN

SHLD 2054

MOV L, C

MOV H, B

SHLD 2055

HLT

INPUT:-

| Address | Data |
| --- | --- |
| 2050 | 10 |
| 2052 | 5 |

OUTPUT:-

| Address | Data |
| --- | --- |
| 2054 | 50 |
| 2055 | 5 |

RESULT:-

Thus the program was executed successfully using 8085 processor simulator.

16-BIT DIVISION

EXP NO: 8

AIM:-

To write an assembly language program to implement 16-bit division using 8085 processor.

ALGORITHM:-

1) Read dividend (16 bit)

2) Read divisor

3) count <- 8

4) Left shift dividend

5) Subtract divisor from upper 8-bits of dividend

6) If CS = 1 go to 9

7) Restore dividend

8) Increment lower 8-bits of dividend

9) count <- count - 1

10) If count = 0 go to 5

11) Store upper 8-bit dividend as remainder and lower 8-bit as quotient

12) Stop

PROGRAM:-

LDA 8500

MOV B,A

LDA 8501

MVI C,00

LOOP: CMP B

JC LOOP1

SUB B

INR C

JMP LOOP

LOOP1: STA 8502

MOV A,C

STA 8503

HLT

INPUT:-

| Address | Data |
| --- | --- |
| 8051 | 20 |
| 8050 | 2 |

OUTPUT:-

| Address | Data |
| --- | --- |
| 8502 | 10 |
| 8503 | 2 |

RESULT:-

Thus the program was executed successfully using 8085 processor simulator

16-BIT ADDITION

EXP NO: 9

AIM :- To write an assembly language program to implement 16-Bit addition using 8086 processor.

ALGORITHM:-

- Start the program by loading a register pair with address of 1st number.

- Copy the data to another register pair.

- Load the second number to the first register.

- Add the two register pair contents.

- Check for carry.

- Store the value of sum and carry in memory location. Result stored in AX. 7-Terminate the program.

PROGRAM :

MOV AX,[1100H]

MOV BX,[1102H]

ADD AX,BX

MOV [1200H], AX

HLT

INPUT :-

| REGISTER | MEMORY | DATA |
| --- | --- | --- |
| AX | 32 | 1100 |
| BX | 45 | 1102 |

OUTPUT :-

| REGISTER | MEMORY | DATA |
| --- | --- | --- |
| AX | 77 | 1200 |

RESULT :- Thus the program was executed successfully using 8086 process simulator.

16 BIT SUBTRACTION

EXP NO: 10

AIM:

To write an assembly language program to implement 16 bit subtraction using 8086 processor.

ALGORITHM:

1] Start the program by loading a register pair with address of first number.

2] Copy the data to another register pair.

3] Laod the second number to first register pair.

4] Subtract the two register pair contacts.

5] Check for borrow.

6] Store the value of difference and borrow in memory location.

7] End.

PROGRAM:

MOV AX,[1100H]

MOV BX,[1102H]

SUB AX,BX

MOV [1200H], AX

HLT

INPUT:

| ADDRESS | DATA |
| --- | --- |
| 1100 | 30 |
| 1102 | 15 |

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 1200 | 15 |

RESULT:

Thus the program was executed successfully using 8086 processor simulator.

16-bit multiplication

EXP NO: 11

Aim: To write an assembly language program to implement 16-bit multiplication on 8086 processer.

ALGORITHM:

- Load the first data in HL pair

- Move content of HL pair to stack pointer

- Load the second data in the HL pair and move it to DE

- Make H register as OH and L register OH

- Add HL pair and stack pointer

- Check for carry if carry increment by 1 else move to next step

- Then move E to A and perform or operation with accumulation and register D

- The value of operation is zero the solve the value else go to step 3

PROGRAM:

MOV AX, [1100H]

MOV BX, [1102H]

MUL BX

MOV [1200H], AX

MOV [1202H], DX

HLT

INPUT:

| ADDRESS | DATA |
| --- | --- |
| 1100 | 20 |
| 1202 | 3 |

OUT PUT:

| ADDRESS | DATA |
| --- | --- |
| 1200 | 60 |
| 1202 | 3 |

RESULT:

Thus the program was executed successfully using 8086 emulator.

16 BIT DIVISION

EXP NO: 12

AIM:

To write an assemble language program to implement 16 bit divided using 8086 processor.

ALGORITHM:

1] Read dividend (16) bit.

2] Read divisor.

3] Count <-8.

4] Left shift dividend.

5] Subtract divisor from upper 8 bits of dividend.

6] If cs=1 go to 9.

7] Restore dividend.

8] Increment lower 8 bits of dividend.

9] Count <- count -1.

10] If count =0 go to 5.

11] Store upper 8 bit dividend as remainder and lower 8 bit as quotient.

12] Stop.

PROGRAM:

MOV AX, [1100H]

MOV BX, [1102H]

DIV BX

MOV [1200H], AX

MOV [1202H], DX

HLT

INPUT:

| ADDRESS | DATA |
| --- | --- |
| 1100 | 10 |
| 1102 | 10 |

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 1200 | 1 |
| 1202 | 10 |

RESULT:

Thus the program was executed successfully using 8086 processor simulator.

Greatest of 2 numbers

EXP NO: 13

Exp. No :-

AIM:-

To write an Assembly Language Program to find the smallest number in an array using 8085 Microprocessor in GNUSim.

SOFTWARE USED:-

GNUSim8085

ALGORITHM:-

1. Initialize the count

2. Get the input numbers

3. Compare the content of Accumulator(A) with HL pair for all input numbers

4. Stores the smallest number in the output register

5. End the program

PROGRAM:-

LDA 2050

MOV B, A

LDA 2051

CMP B

JNC STORE

MOV A, B

STORE: STA 2052

HLT

Input

| Address | Data |
| --- | --- |
| 2050 | 29 |
| 2051 | 22 |

Output

| Address | Data |
| --- | --- |
| 2052 | 29 |

RESULT:

Thus the Assembly Language Program to find the smallest number in an array using 8085 Microprocessor in GNUSim is performed.

Smallest of 2 numbers

EXP NO: 14

AIM:-

To write an Assembly Language Program to find the smallest number in an array using 8085 Microprocessor in GNUSim.

SOFTWARE USED:-

GNUSim8085

ALGORITHM:-

1. Initialize the count

2. Get the input numbers

3. Compare the content of Accumulator(A) with HL pair for all input numbers

4. Stores the smallest number in the output register

5. End the program

PROGRAM:-

LXI H,8050

MOV C, M

INX H

MOV B, M

DCR C

LOOP: INX H

MOV A, M

CMP B

JNC SKIP

MOV B, A

SKIP: DCR C

JNZ LOOP

LXI H,8500

MOV M, B

HLT

Input

| Address | Data |
| --- | --- |
| 8000 | 10 |

Output

| Address | Data |
| --- | --- |
| 8010 | 3 |

RESULT:

Thus the Assembly Language Program to find the smallest number in an array using 8085 Microprocessor in GNUSim is performed.

SWAPING OF TWO 8-BIT DATA

EXP NO: 15

AIM:

To Write an assembly language program to swap two 8-bit data using 8085 processor.

ALGORITHM:

1. Load the contents of memory address 1100 into accumulator A.

2. Move the contents of accumulator A into register B.

3. Load the contents of memory address 1101 into accumulator A.

4. Move the contents of accumulator A into register C.

5. Store the contents of accumulator A (which is the original value at 1101) into memory address 1102.

6. Move the contents of register B (which is the original value at 1100) into accumulator A.

7. Store the contents of accumulator A into memory address 1103.

PROGRAM:

LDA 1100

MOV B, A

LDA 1101

MOV C, A

STA 1102

MOV A, B

STA 1103

HLT

INPUT:

| ADDRESS | DATA |
| --- | --- |
| 1100 | 6 |
| 1101 | 4 |

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 1103 | 4 |
| 1104 | 6 |

RESULT:

Thus the program was executed successfully using 8085 processor simulator.

1’s COMPLIMENT

EXP NO: 16

AIM :

To write assembly language to find 1’s COMPLIMENT by using 8085 microprocessor Simulator

ALGORITHM:

1. Loads the value from memory address 8000 into accumulator A.

2. Complements the bits of the value in accumulator A using the CMA (Complement Accumulator) instruction. This means that all 1s become 0s and all 0s become 1s.

3. Stores the complemented value into memory address 8001.

4. Halts the program execution.

PROGRAM:

LDA 8000

CMA

STA 8001

HLT

INPUT:

| ADDRESS | DATA |
| --- | --- |
| 8000 | 6 |

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 8001 | 249 |

RESULT: THIS PROGRAM WAS EXECUTED SUCCESSFULLY BY USING 8085 MICROPROCESSOR SIMULATOR

2’S COMPLEMENT

EXP NO: 17

AIM:

To write an assembly language program to find 2’s complement of 8-bit number

ALGORITHM:

- Start with the binary number:

- If the number is positive, simply write its binary equivalent.

- If the number is negative, begin with the binary equivalent of its positive value.

- Invert all the bits (1's complement):

- Flip every 0 to 1 and every 1 to 0

- Add 1 to the result:

- Add 1 to the least significant bit (rightmost bit) of the inverted number.

- The final result is the 2's complement representation of the number.

PROGRAM:

LDA 3000

CMA

STA 3001

ADI 3002

HLT

INPUT:

| ADDRESS | DATA |
| --- | --- |
| 3000 | 8 |

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 3001 | 247 |
| 3002 | 0 |

RESULT: Thus the PROGRAM WAS EXECUTED SUCCESSFULLY USING 8085 PROCESSOR SIMULATER

ODD OR EVEN – 8085 MICROPROCESSOR

EXP NO: 18

AIM:

To write an assembly language program to find the number is odd or even using 8085 Microprocessor in GNUSim8085

SOFTWARE USED:-

GNUSim8085

ALGORITHM:-

- Initialize the number in the accumulator

- Perform the AND operation with accumulator by 01

- If the result is ‘0’, it means it is even number (indicates as 22)

- If the result is non zero , it means the given number is odd (indicates as 11)

- Stores the out put in the register

- End the program

Program :-

LDA 8050H

ANI 80H

JZ POS

MVI A,11

JMP STO

POS: MVI A,22

STO: STA 8051H

HLT

Input :

| Address | Data |
| --- | --- |
| 8050 | 20 |

| Address | Data |
| --- | --- |
| 8050 | 19 |

Output:

| Address | Data |
| --- | --- |
| 8051 | 22 |

| Address | Data |
| --- | --- |
| 8051 | 11 |

RESULT:-

Thus the assembly Language Program to find the ODD OR EVEN is performed using 2050H Microprocessor in GNUSim8085

POSITIVE AND NEGATIVE

EXP.NO : 19

AIM:

To write an assembly language program to find the number is POSITIVE AND NEGATIVE using 8085 Microprocessor in GNUSim

SOFTWARE USED:-

GNUSim 8085

ALGORITHM:-

- Initialize the number in the accumulator

- Perform the AND operation with accumulator by 01

- If the result is ‘0’, it means it is even number (indicates as 22)

- If the result is non zero , it means the given number is odd (indicates as 11)

- Stores the out put in the register

- End the program

Program :-

LDA 8050

ANI 01H

JZ LOOP1

MVI A, 11

JMP LOOP2

LOOP1: MVI A, 22

LOOP2: STA 8051

HLT

OUT PUT :-

Input :

| Address | Data |
| --- | --- |
| 8050 | 0 |

| Address | Data |
| --- | --- |
| 8050 | 15 |

Output:

| Address | Data |
| --- | --- |
| 8051 | 22 |

| Address | Data |
| --- | --- |
| 8051 | 11 |

RESULT:-

Thus the assembly Langua6g36e5 885250Program to find the positive or negative is performed using 2050H Microprocessor in GNUSim

ASCENDING  ORDER  – 8085 MICROPROCESSOR

EXP NO: 20

AIM:

To write an assembly language program to find the ascending order of numbers using 8085 Microprocessor in GNUSim8085

SOFTWARE USED:-

GNUSim8085

ALGORITHM:-

- Initialize the count

- Get the input numbers

- compare content accumulator [A] with HL pair for all input numbers

- stores the ascending numbers in the output registers

- end the program

Program :-

LXI H,8000

MOV C,M

DCR C

LOOP3: MOV D,C

LXI H,8001

LOOP2: MOV A,M

INX H

CMP M

JC LOOP1

MOV B,M

MOV M,A

DCX H

MOV M,B

INX H

LOOP1: DCR D

JNZ LOOP2

DCR C

JNZ LOOP3

HLT

Input :

| Address | Data |
| --- | --- |
| 8000 | 3 |
| 8001 | 4 |
| 8002 | 18 |

Output:

| Address | Data |
| --- | --- |
| 8001 | 3 |
| 8002 | 4 |
| 8003 | 18 |

RESULT:            Thus the assembly Language Program to find the Ascending order of numbers is performed using 8085 Microprocessor in GNUSim8085

DESCENDING ORDER

EXP NO: 21

AIM:-

To write an assembly language program to implement descending order using 8085 processor.

ALGORITHM:-

1)  Load the number of elements in the array (N) into a register.

2) Use nested loops:

Outer loop: Decrease the range of comparison in each iteration.

Inner loop: Compare adjacent elements and swap if needed.

3) Repeat until the array is sorted in descending order.

PROGRAM:-

LXI H,8050

MOV C,M

DCR C

LOOP3: MOV D,C

LXI H,8051

LOOP2: MOV A,M

INX H

CMP M

JNC LOOP1

MOV B,M

MOV M,A

DCX H

MOV M,B

INX H

LOOP1: DCR D

JNZ LOOP2

DCR C

JNZ LOOP3

HLT

INPUT:-

| Address | Data |
| --- | --- |
| 2001 | 2 |
| 2002 | 6 |
| 2003 | 3 |
| 2004 | 2 |
| 2005 | 5 |

OUTPUT:-

| Address | Data |
| --- | --- |
| 2010 | 120 |

RESULT:-

Thus the program was executed successfully using 8085 processor simulator.

LARGEST NUMBER IN AN ARRAY

EXP NO: 22

AIM:

To write an Assembly Language Program to find the largest number in an array

using 8085 Microprocessor in GNUSim.

SOFTWARE USED:

GNUSim8085

ALGORITHM:

- Initialize the count

- Get the input numbers

- Compare the content of Accumulator(A) with HL pair for all input numbers

- Stores the largest number in the output register

- End the program

PROGRAM:

LXI H,8050

MOV C, M

INX H

MOV B, M

DCR C

LOOP: INX H

MOV A, M

CMP B

JC SKIP

MOV B, A

SKIP: DCR C

JNZ LOOP

LXI H,8500

MOV M, B

HLT

Input

| Address | Data |
| --- | --- |
| 8050 (Counter) | 5 |

| Address | Data |
| --- | --- |
| 8051 | 5 |
| 8052 | 2 |
| 8053 | 6 |
| 8054 | 8 |
| 8055 | 9 |

Output:

| Address | Data |
| --- | --- |
| 8500 | 9 |

RESULT:

Thus the Assembly Language Program to find the largest number in an array using 8085 Microprocessor in GNUSim is performed.

SMALLEST NUMBER IN AN ARRAY

EXP NO: 23

AIM:

To write an Assembly Language Program to find the smallest number in an array

using 8085 Microprocessor in GNUSim.

SOFTWARE USED:

GNUSim8085

ALGORITHM:

- Initialize the count

- Get the input numbers

- Compare the content of Accumulator(A) with HL pair for all input numbers

- Stores the smallest number in the output register

- End the program

PROGRAM:

LXI H, 2050H

MOV C, M

INX H

MOV A, M

DCR C

LOOP: INX H

CMP M

JC NEXT

MOV A, M

NEXT: DCR C

JNZ LOOP

STA 3052H

HLT

| Address | Data |
| --- | --- |
| 8050 (Counter) | 5 |

| Address | Data |
| --- | --- |
| 8051 | 2 |
| 8052 | 4 |
| 8053 | 7 |
| 8054 | 5 |
| 8055 | 9 |

Output:

| Address | Data |
| --- | --- |
| 8500 | 2 |

RESULT:

Thus the Assembly Language Program to find the smallest number in an array using 8085 Microprocessor in GNUSim is performed.

LCM – 8085 MICROPROCESSOR

EXP NO: 24

AIM:

To write an assembly language program to find the LCM of numbers using 8085 Microprocessor in GNUSim8085

SOFTWARE USED:-

GNUSim8085

ALGORITHM:-

- start the program.

- Load A into the accumulator.

- Move A to R1.

- Load B into the accumulator.

- Move B to R2.

- Call the GCD subroutine (the GCD subroutine is already implemented using the Euclidean algorithm).

- Compute Product:

- Multiply A and B to get the product.

- Store this product temporarily.

- Divide the Product by GCD to get the LCM.

- Store the LCM at a memory location (e.g., 6009).

- Halt the program

PROGRAM:-

Input :

| Address | Data |
| --- | --- |
| 8000 | 60 |
| 8001 | 45 |

Output:

| Address | Data |
| --- | --- |
| 8011 | 225 |

RESULT:            Thus the assembly Language Program to find the LCM of numbers is performed using 8085 Microprocessor in GNUSim8085

GCD

EXP NO: 25

AIM:

To write an assembly language program to implement GCD using 8085 processor.

ALGORITHM:

- Start the program by loading the first number (A) into the accumulator.

- Move the first number (A) to a register (R1) to store it temporarily.

- Get the second number (B) and load it into the accumulator.

- Compare if B is greater than 0 (i.e., check if the divisor is non-zero).

- Perform division of A by B and calculate the remainder.

- If the remainder is 0, the GCD is B. Store the result in memory (at a designated location).

- If the remainder is not 0, move B to register R1 and load the remainder into the accumulator.

- Repeat the steps from step 4 (looping back) until the remainder becomes 0.

- Store the result (GCD) when the loop terminates and the remainder is 0.

- Halt the program after completing the process.

PROGRAM:

INPUT:

| ADDRESS | DATA |
| --- | --- |
| 6000 | 38 |
| 6001 | 19 |

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 6009 | 19 |

RESULT: Thus the program was executed successfully using 8085 processor simulator.

Factorial

EXP NO: 26

AIM :

To Write an assembly language program to find factorial of n in the given

ALGORITHM:

- Load the address 8050H into the HL register pair.

- Move the value from the memory location (8050H) into the B register.

- Load the value 01H into the D register to serve as an accumulator for the factorial result.

- Call the subroutine MUL to multiply the current value of D (partial factorial) by B.

- Decrement the B register to move to the next value in the factorial computation.

- Check if B is zero. If not, jump back to the label FACT.

- Increment the HL register to point to the next memory location (8051H).

- Store the result from the D register at the memory location pointed to by HL.

- Halt the program.

- Move the current value of B into the E register (as a multiplier).

- Clear the A register (set it to 0) to use as a running total for the multiplication.

- Perform repeated addition (ADD D) E times to compute the product.

- Decrement the E register after each addition and check if E is zero.

- When E becomes zero, move the result from A to D and return.

PROGRAM:

LXI H,8000

MOV C,M

MVI B,00

INX H

MOV B,M

CMA

MOV E,A

MVI D,00FH

MOV A,B

CMA

MOV D,A

INX D

LXI H,0000

NEXT: DAD B

SHLD 8010

LOOP: DAD D

JNC SKIP

MOV A,H

ORA L

JZ EXIT

JMP LOOP

SKIP: LHLD 8010

JMP NEXT

EXIT: LHLD 8010

HLT

| ADDRESS | DATA |
| --- | --- |
| 8050 | 5 |

INPUT:

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 8051 | 120 |

RESULT:  Thus the program was executed successfully using 8085 processor simulator.

DECIMAL TO HEXA DECIMAL

EXP NO: 27

AIM:  Write a program to convert Decimal number to Hexadecimal number

SOFTWARE : GNUSIM 8085

ALGORITHM:

- Initialize Registers:

- Store the decimal number in a register (e.g., register B).

- Perform repeated division of the decimal number by 16 to obtain the hexadecimal digits.

- Store the quotient in a register (e.g., B or C).

- Store the remainder (hex digit) separately.

- If the remainder is greater than 9, convert it to its corresponding ASCII representation for A-F (e.g., add 7 to the remainder).

- Store the hexadecimal digits (remainders) in reverse order in memory.

- If the quotient is zero, the conversion is complete. Otherwise, repeat the division step with the quotient as the new dividend.

- Use the stored hexadecimal digits to display the result.

PROGRAM:

INPUT:

| ADDRESS | DATA |
| --- | --- |
| 2050 | 34 |

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 2051 | 0 |
| 2052 | 84 |

RESULT: Thus the PROGRAM WAS EXECUTED SUCCESSFULLY USING 8085 PROCESSOR SIMULATER